Model: PC-010

RS485 & Counter - Websocket

PCB 140166-03-v4.1

PCB Board

Circuit Diagram

Features:

# ESP32 RS485 Dashboard - Complete Feature Overview

## 🌐 Web Interface Features

### **Dashboard Page** (`/`)

- **Live RS485 Data Display**: 4 sensor values with customizable labels and scaling

- **Pulse Counter Display**: Shows Qty (current), Accm (accumulated), and CPM (pulses per minute)

- **Real-time Updates**: Auto-refresh every 10 seconds

- **Status Indicators**: Connection status and last send status

- **Navigation Links**: Settings, WiFi Setup, and manual send button

### **Settings Page** (`/settings`)

- **Device Configuration**: Device ID, Machine ID, Send interval

- **Remote Server Settings**: Host, RS485 path, Counter path

- **RS485 Parameters**: Slave address, start register, count, baud rate

- **Scaling Factors**: K1-K4 for sensor value calibration

- **Custom Labels**: V1-V4 sensor names (max 6 chars)

- **Pulse Configuration**: Pulse divider (default 10)

- **Dashboard Header**: Customizable page title (max 30 chars)

- **Enable/Disable Controls**:

- Counter Status (Enabled/Disabled)

- RS485 Status (Enabled/Disabled)

- **Counter Reset Buttons**:

- Reset Qty & Accm (both counters)

- Reset Qty Only (current count)

- Reset Accm Only (accumulated total)

- **System Control**: Reboot device button

### **WiFi Setup Page** (`/setup`)

- **AP Mode**: Creates hotspot "ESP32-RS485-Setup" (password: 12345678)

- **Network Scanner**: Shows available WiFi networks

- **Configuration**: SSID, password, static/DHCP IP settings

- **Auto-reboot**: Applies settings and restarts device

## 🔧 Hardware Features

### **RS485 Modbus Communication**

- **Hardware**: UART on GPIO26 (RX) and GPIO27 (TX)

- **Configurable**: Slave address, register range, baud rate

- **Error Handling**: CRC validation, timeout protection, retry logic

- **Debug Output**: Raw hex data for troubleshooting

### **Pulse Counter System**

- **Input**: GPIO25 with 30ms debouncing

- **Counters**:

- `pulse_count`: Total pulses since boot

- `pulse_accm`: Accumulated total (persists)

- `cpm`: Current pulses per minute

- **Interrupt-driven**: Efficient counting using hardware interrupts

- **Power Failure Protection**: Pin 34 with 2k-2k divider saves counters on power loss

### **LCD Display** (I2C 1602)

- **Auto-cycling Pages**:

- Page 0: Time + IP address

- Page 1: RS485 sensor values

- Page 2: Pulse metrics

- **I2C Addresses**: Auto-detects 0x27 or 0x3F

- **Pins**: SCL=GPIO22, SDA=GPIO21

## 📊 Data Management

### **Remote Server Integration**

- **RS485 Data**: `http://host/iot2025/e089/insertM.php?devid=...&v1=...&v2=...&v3=...&v4=...`

- **Pulse Data**: `http://host/iot2025/e089/insert2C.php?devid=...&mcid=...&qty=...&accm=...&cpm=...`

- **Configurable**: Remote host and paths

- **URL Debug**: Prints full URLs before sending

### **Local Data Logging**

- **Pulse Log**: `pulse_log.txt` with 200-record circular buffer

- **Format**: `timestamp,qty,accm` (newest records first)

- **Auto-rotation**: Removes oldest when exceeding 200 records

### **Configuration Persistence**

- **File**: `rs485_device_config.txt` (25 lines)

- **Stores**: All settings, counters, enable/disable states

- **Auto-save**: On settings change, pulse divider reached, power failure

- **Boot Load**: Restores all previous state

## 🔄 Operational Features

### **Pulse Divider System**

- **Function**: Sends data every N pulses (default 10)

- **Auto-reset**: Divider counter resets after send

- **Configurable**: 1-10000 pulses per divider

### **Enable/Disable Controls**

- **Counter Disable**: Stops counting, sending, and logging

- **RS485 Disable**: Stops sensor reading and sending

- **Persistent**: Settings saved across reboots

### **Power Management**

- **Power Failure Detection**: Pin 34 monitors main power

- **Emergency Save**: Immediately saves counters on power loss

- **Graceful Recovery**: Restores state on power return

### **WiFi Management**

- **Dual Mode**: Station mode (client) or AP mode (hotspot)

- **Fallback**: Auto-switches to AP if WiFi fails

- **Static/DHCP**: Configurable IP settings

- **Recovery**: AP mode for initial setup

## 🛠️ Technical Features

### **Error Handling**

- **Network**: Timeout protection, retry logic

- **File I/O**: Exception handling for config operations

- **Hardware**: Pin initialization fallbacks

- **Communication**: CRC validation for RS485

### **Performance**

- **Memory**: Garbage collection every 60 seconds

- **Interrupts**: Deferred processing for pulse counting

- **Non-blocking**: Web server handles multiple clients

- **Efficient**: Minimal CPU usage during idle periods

### **Debug Features**

- **Serial Output**: Detailed status messages

- **URL Logging**: Full request URLs printed

- **Error Codes**: Specific error reporting

- **Status Indicators**: Visual feedback on web interface

## 📱 Mobile Compatibility

- **Responsive Design**: Works on mobile browsers

- **Touch-friendly**: Large buttons and controls

- **Auto-refresh**: Live data updates

- **AP Access**: Direct connection in setup mode

This system provides a complete industrial monitoring solution with pulse counting, sensor data collection, web interface, and robust data persistence - all configurable through a simple web interface.

Web-Server

Dashboard:

Settings:

WiFi Setup